###### FOMADUX NDT ROLLFILM

TECHNICAL RADIOGRAPHIC FILMS

# Characteristics of the product

FOMADUX NDT ROLLFILM is a specially produced format of industrial radiographic film designed for non destructive testing with use of X- or gamma radiation.

The film is packed in a special outer paper sleeve which ensures the film’s resistance to light, humidity and greasy impurities. The screens are lacquered to prevent oxidation or degredation in extreme climates.

The following types of technical radiographic film are supplied as FOMADUX NDT ROLLFILM:

INDUX R4 with or without lead screens

INDUX R5 with or without lead screens

INDUX R7 with or without lead screens

# Advantages of the rollfilm

The full length of the weld can be radiographed onto a single piece of film, i.e. this avoids using several film sheets for one weld. Required film length is determined according to the weld length.

 It avoids using separate cassettes and screens and time consuming loading of each film in a darkroom.

 Single- use lead screens avoid faults which can arise    with multi -use screens and cassettes

 Lightproof package is resistant to humidity and greasy impurities.

 It is convenient to use in daylight or adverse  conditions on remote sites.

 Excellent contact between the film, lead screens and the object being checked ensures optimal quality of the image.

 The "edge to edge" packing of the film in the outer sleeve allows maximum use of the film surface where there is restricted space around the weld to place the film.

# Usage

The advantage of the rollfilm is, in particular, the increased efficiency of being able to make one exposure instead of multiple ones when examining long welds. e.g. when radiographing welds of pipelines, pressure vessels or large parts in the aerospace industry, the required length can be chosen so that all the radiogram can be exposed to a single piece of film. Due to the outer paper sleeve there is no contact of the film with a wet or impure object and therefore no adverse effect on the quality of the final radiograph.

This product is manufactured and marketed in conformity with a quality system according to the international standard  ISO 9001.

FORMATS

FOMADUX NDT ROLLFILM is supplied in the following sizes :

Film dimensions:	width:	70 and 100 mm

length:	up to 90 m

# Packing:

a) With intensifying lead screens

The  film  is  placed between two lead screens (thickness 0,025 mm) of the same dimensions; the screens are in perfect contact with the film; the film is packed in a light- and moisture-proof package – using the "edge to edge" system. Packed film is wound onto a cardboard spool and placed in a cardboard transport box from which the required length can be simply wound out.

b) Without lead screens

Same packing as in a), without screens.

At intervals on the upper,sealed side of the film there are printed lines which help determine the film centre and the film length.

METHOD OF REMOVING FILM FROM OUTER SLEEVE BEFORE PROCESSING :

In a dark room hold the non-printed part of the sleeve together with the lead screen and film with one hand, and strip off the printed part of the outer paper sleeve and the second lead screen with the other hand. In this way,the film will be easily and quickly removed from the package.

# Processing

Chemicals

The same chemicals as for other industrial radiographic INDUX films can be used.

# Processing technology

Long lengths of films must be processed carefully. Generally it is possible to cut the film after marking it and then to process it in the ordinary way,or to fully exploit the advantages of the ROLLFILM package it can be processed in a single length.

- When machine processing it is necessary to ensure the film is accurately and squarely fed  (with the help of a suitable guide ,etc) into the the automatic processor.

- When manually processing  ROLLFILM, prior to processing it is necessary to wind the film into a special wire coil that facilitates contact of the film with baths. It is also possible to cut the film and to process it in parts loaded in to hangers used for processing sheet films.

| #### FOMA |  |  | FOMA 11/00 |
| --- | --- | --- | --- |
|  | 501 04 Hradec Králové | Tel.: +420–49–61 40 02 | e-mail: export@foma.cz |
| Bohemia Ltd. | Czech Republic | Fax: +420–49–61 39 55 | http://www.foma.cz |